EXP.2. Numerical Simulation and Performance Analysis of Microwave Transmission Lines and Two-Port Networks Using Scattering Parameters

A.

# Code:

clear; clc; close all;

% Given parameters

Z0 = 50;	% Port reference impedance (ohms)

Zc = 50;	% Line characteristic impedance (ohms) L = 50e-3;		% Line length (m)

f = linspace(8e9,12e9,1001);

f0 = 10e9;	% Operating frequency (Hz) c = 299792458;		% Speed of light (m/s)

% Ideal lossless transmission line beta = 2*pi*f/c;

gamma = 1i*beta;

% ABCD parameters

A = cosh(gamma*L);

B = Zc.*sinh(gamma*L);

C = sinh(gamma*L)./Zc;

D = A;

% ABCD-to-S parameter conversion den = A + B/Z0 + C*Z0 + D;

S11 = (A + B/Z0 - C*Z0 - D)./den;

S21 = 2./den;

S12 = 2.*(A.*D - B.*C)./den;

S22 = (-A + B/Z0 - C*Z0 + D)./den;

% Find the frequency index nearest to 10 GHz [~,idx] = min(abs(f-f0));

% Calculate results at 10 GHz

RL_dB = -20*log10(max(abs(S11(idx)),eps)); IL_dB = -20*log10(abs(S21(idx)));

efficiency = abs(S21(idx))^2*100;

fprintf('Results at %.2f GHz\n',f(idx)/1e9);

fprintf('S11 = %.4f %+.4fj\n',real(S11(idx)),imag(S11(idx))); fprintf('S12 = %.4f %+.4fj\n',real(S12(idx)),imag(S12(idx))); fprintf('S21 = %.4f %+.4fj\n',real(S21(idx)),imag(S21(idx))); fprintf('S22 = %.4f %+.4fj\n',real(S22(idx)),imag(S22(idx))); fprintf('Return loss = %.2f dB\n',RL_dB);

fprintf('Insertion loss = %.2f dB\n',IL_dB);

fprintf('Transmission efficiency = %.2f %%\n',efficiency);

% Plot S-parameter magnitudes figure;

plot(f/1e9,20*log10(max(abs(S11),eps)),'LineWidth',1.5); hold on;

plot(f/1e9,20*log10(abs(S21)),'LineWidth',1.5); plot(f/1e9,20*log10(abs(S12)),'--','LineWidth',1.5);

plot(f/1e9,20*log10(max(abs(S22),eps)),'--','LineWidth',1.5); grid on;

xlabel('Frequency (GHz)'); ylabel('Magnitude (dB)');

title('S-Parameters of a 50 \Omega Transmission Line'); legend('S11','S21','S12','S22','Location','best');

### Output:

B.

## code:

% Find minimum |S11| and maximum |S21| [~,idxMinS11] = min(abs(S11));

[~,idxMaxS21] = max(abs(S21));

% Report the frequency for each optimum fprintf('\nPart B: Optimum Frequency\n');

fprintf('Frequency of minimum |S11| = %.6f GHz\n', ... f(idxMinS11)/1e9);

fprintf('S11 at this frequency = %.6f %+.6Ƞ\n', ... real(S11(idxMinS11)),imag(S11(idxMinS11)));

fprintf('Return loss there = %.3f dB\n', ...

-20*log10(max(abs(S11(idxMinS11)),eps)));

fprintf('\nFrequency of maximum |S21| = %.6f GHz\n', ... f(idxMaxS21)/1e9);

fprintf('S21 at this frequency = %.6f %+.6Ƞ\n', ... real(S21(idxMaxS21)),imag(S21(idxMaxS21)));

fprintf('Insertion loss there = %.3f dB\n', ...

-20*log10(abs(S21(idxMaxS21))));

% Compare with the values at 10 GHz fprintf('\nComparison with 10 GHz:\n');

fprintf('At 10 GHz: |S11| = %.6g, |S21| = %.6g\n', ... abs(S11(idx)),abs(S21(idx)));

fprintf('At optimum: |S11| = %.6g, |S21| = %.6g\n', ... abs(S11(idxMinS11)),abs(S21(idxMaxS21)));

Output: