CO2

SIMULATION EXPERIMENT – 5

Electromagnetic Simulation and	S-Parameter Characterization of	a Microwave Directional Coupler and Two-Way Power Divider.

A.	Directional Coupler and Divider: Design a 10 GHz microwave directional coupler with 10 dB coupling and a two-way 3 dB power divider, both using 50 Ω ports. Simulate from 8–12 GHz. At 10 GHz obtain S11, S21, S31 and the isolated-port response. Calculate coupling, isolation, insertion loss, directivity and power-division ratio and compare the results with 10 dB/3 dB design targets.

B.	Optimization: Change only the coupling-section length by 5% from its initial value and repeat the 8–12 GHz simulation. Compare coupling, isolation, directivity, insertion loss and S-parameter curves before and after optimization. Identify the design that maintains the closest 10 dB coupling and lowest insertion loss over 9–11 GHz.

Code:

%% Experiment 5: Directional Coupler and Two-Way Divider

clear; clc; close all;

f = linspace(8e9, 12e9, 801);

fGHz = f / 1e9;

f0 = 10e9;

% Illustrative coupler model, set for 10 dB coupling at 10 GHz.

% The +5% change modifies the electrical length.

theta = (pi/2) * f/f0;

theta5 = 1.05 * theta;

targetCoupledPower = 10^(-10/10);

coupledPower = targetCoupledPower .* sin(theta).^2;

coupledPower5 = targetCoupledPower .* sin(theta5).^2;

S31_dB = 10*log10(max(coupledPower, eps));

S31_5_dB = 10*log10(max(coupledPower5, eps));

S21_dB = 10*log10(max(1 - coupledPower, eps));

S21_5_dB = 10*log10(max(1 - coupledPower5, eps));

% Illustrative return loss and isolated-port response

S11_dB = -30 + 20*log10(1 + abs(f-f0)/1e9);

S41_dB = -30 + 2*abs(fGHz - 10);

[~, i10] = min(abs(f-f0));

coupling = -S31_dB(i10);

isolation = -S41_dB(i10);

insertionLoss = -S21_dB(i10);

directivity = isolation - coupling;

fprintf('At 10 GHz:\\n');

fprintf('S11 = %.2f dB, S21 = %.2f dB, S31 = %.2f dB, S41 = %.2f dB\\n', ...

S11_dB(i10), S21_dB(i10), S31_dB(i10), S41_dB(i10));

fprintf('Coupling = %.2f dB; isolation = %.2f dB\\n', coupling, isolation);

fprintf('Insertion loss = %.2f dB; directivity = %.2f dB\\n', ...

insertionLoss, directivity);

% Ideal matched, equal two-way divider

dividerS21_dB = 10*log10(0.5);

dividerS31_dB = 10*log10(0.5);

fprintf('\\nIdeal 3 dB divider: S21 = %.2f dB, S31 = %.2f dB; ratio = 1:1\\n', ...

dividerS21_dB, dividerS31_dB);

% S-parameter curves

figure;

plot(fGHz, S11_dB, fGHz, S21_dB, fGHz, S31_dB, fGHz, S41_dB, ...

'LineWidth', 1.5);

grid on; xlabel('Frequency (GHz)'); ylabel('Magnitude (dB)');

title('Directional Coupler S-Parameters');

legend('S11','S21','S31 coupled','S41 isolated','Location','best');

% Coupling before and after the 5% length change

figure;

plot(fGHz, S31_dB, 'LineWidth', 1.5); hold on;

plot(fGHz, S31_5_dB, '--', 'LineWidth', 1.5);

yline(-10, ':k', '10 dB target');

grid on; xlabel('Frequency (GHz)'); ylabel('S31 (dB)');

title('Coupling Before and After 5% Length Change');

legend('Initial length','Length +5%','Target','Location','best');

% Average comparison over 9-11 GHz

band = f >= 9e9 & f <= 11e9;

fprintf('\\n9-11 GHz average absolute coupling error:\\n');

fprintf('Initial = %.3f dB; +5%% length = %.3f dB\\n', ...

mean(abs(S31_dB(band)+10)), mean(abs(S31_5_dB(band)+10)));

fprintf('Average insertion loss: initial = %.3f dB; +5%% length = %.3f dB\\n', ...

mean(-S21_dB(band)), mean(-S21_5_dB(band)));

output:

SIMULATION EXPERIMENT – 6

Simulation-Based	Investigation	of	Microwave	Semiconductor	Devices	and Frequency-Conversion Characteristics of a Microwave Mixer

Objectives of this Experiment:

1.	To develop simulation models representing the operating characteristics of selected microwave semiconductor devices.

2.	To analyze the electrical characteristics and frequency-dependent behavior of Gunn, Schottky or PIN diode-based microwave structures.

3.	To simulate frequency conversion in a microwave mixer and identify the generated frequency components.

4.	To evaluate conversion characteristics and identify suitable operating conditions for microwave applications.

Experimental Procedure

A.	Mixer Frequency Conversion: Set fRF = 10 GHz, fLO = 9.5 GHz, PRF = −10 dBm and PLO = +7

dBm with a 50 Ω IF load. Obtain the output spectrum. Calculate the theoretical IF frequency |10−9.5| =

0.5 GHz and sum frequency 19.5 GHz. Identify the simulated components and calculate the percentage frequency deviation.

B.	Conversion Loss: Using the same mixer, record the IF output power at 0.5 GHz. Calculate conversion loss CL = PRF − PIF and compare it with the expected passive-mixer range of approximately 6–10 dB. Explain how RF and LO power affect the observed conversion performance.

Code:

%% Experiment 6: Mixer Frequency Conversion

clear; clc; close all;

% Given mixer settings

fRF = 10e9;

fLO = 9.5e9;

PRF_dBm = -10;

PLO_dBm = 7;

% Theoretical output frequencies

fIF = abs(fRF - fLO);

fSum = fRF + fLO;

fprintf('Theoretical IF frequency = %.3f GHz\\n', fIF/1e9);

fprintf('Theoretical sum frequency = %.3f GHz\\n', fSum/1e9);

% Example spectrum components.

% Replace these illustrative power values with your simulator readings.

spectrumFreq = [fIF, fLO, fRF, fSum];

spectrumPower_dBm = [-17, PLO_dBm, PRF_dBm, -25];

% Example simulated frequency values; replace with measured/simulated peaks.

simIF = 0.500e9;

simSum = 19.500e9;

ifDeviation = 100 * abs(simIF-fIF)/fIF;

sumDeviation = 100 * abs(simSum-fSum)/fSum;

fprintf('\\nSimulated IF = %.3f GHz; deviation = %.4f%%\\n', ...

simIF/1e9, ifDeviation);

fprintf('Simulated sum = %.3f GHz; deviation = %.4f%%\\n', ...

simSum/1e9, sumDeviation);

% Conversion loss using the example IF output power

PIF_dBm = spectrumPower_dBm(1);

conversionLoss_dB = PRF_dBm - PIF_dBm;

fprintf('\\nExample IF output power = %.2f dBm\\n', PIF_dBm);

fprintf('Conversion loss = PRF - PIF = %.2f dB\\n', conversionLoss_dB);

% Plot example output spectrum

figure;

stem(spectrumFreq/1e9, spectrumPower_dBm, 'filled', 'LineWidth', 1.5);

grid on; xlabel('Frequency (GHz)'); ylabel('Power (dBm)');

title('Illustrative Mixer Output Spectrum');

xticks([0.5 9.5 10 19.5]);

xticklabels({'IF: 0.5','LO: 9.5','RF: 10','Sum: 19.5'});

xlim([0 21]);

output: